Week-3

#include <SoftwareSerial.h>

SoftwareSerial Bluetooth(10,9); // RX, TX

int LED = 13;   // the on-board LED

int data;       // the data received

void setup() {

Bluetooth.begin(9600);

Serial.begin(9600);

Serial.println("Waiting for command...");

Bluetooth.println("Send 1 to turn ON LED");

Bluetooth.println("Send 0 to turn OFF");

pinMode(LED, OUTPUT);

}

void loop() {

if (Bluetooth.available()) {

data = Bluetooth.read();

if (data == '1') {

digitalWrite(LED, HIGH);

Serial.println("LED ON");

}

else if (data == '0') {

digitalWrite(LED, LOW);

Serial.println("LED OFF");

}

else {

}

delay(100);

}

}

Week-4

Read prg:

#include <SPI.h>

#include <MFRC522.h>

#define RST_PIN         9           // Configurable, see typical pin layout above

#define SS_PIN          10          // Configurable, see typical pin layout above

MFRC522 mfrc522(SS_PIN, RST_PIN);   // Create MFRC522 instance

//*****************************************************************************************//

void setup() {

Serial.begin(9600);                                           // Initialize serial communications with the PC

SPI.begin();                                                  // Init SPI bus

mfrc522.PCD_Init();                                              // Init MFRC522 card

Serial.println(F("Read personal data on a MIFARE PICC:"));    //shows in serial that it is ready to read

}

//*****************************************************************************************//

void loop() {

// Prepare key - all keys are set to FFFFFFFFFFFFh at chip delivery from the factory.

MFRC522::MIFARE_Key key;

for (byte i = 0; i < 6; i++) key.keyByte[i] = 0xFF;

//some variables we need

byte block;

byte len;

MFRC522::StatusCode status;

//-------------------------------------------

// Reset the loop if no new card present on the sensor/reader. This saves the entire process when idle.

if ( ! mfrc522.PICC_IsNewCardPresent()) {

return;

}

// Select one of the cards

if ( ! mfrc522.PICC_ReadCardSerial()) {

return;

}

Serial.println(F("**Card Detected:**"));

//-------------------------------------------

mfrc522.PICC_DumpDetailsToSerial(&(mfrc522.uid)); //dump some details about the card

//mfrc522.PICC_DumpToSerial(&(mfrc522.uid));      //uncomment this to see all blocks in hex

//-------------------------------------------

Serial.print(F("Name: "));

byte buffer1[18];

block = 4;

len = 18;

//------------------------------------------- GET FIRST NAME

status = mfrc522.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, 4, &key, &(mfrc522.uid)); //line 834 of MFRC522.cpp file

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Authentication failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

status = mfrc522.MIFARE_Read(block, buffer1, &len);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Reading failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

//PRINT FIRST NAME

for (uint8_t i = 0; i < 16; i++)

{

if (buffer1[i] != 32)

{

Serial.write(buffer1[i]);

}

}

Serial.print(" ");

//---------------------------------------- GET LAST NAME

byte buffer2[18];

block = 1;

status = mfrc522.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, 1, &key, &(mfrc522.uid)); //line 834

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Authentication failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

status = mfrc522.MIFARE_Read(block, buffer2, &len);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Reading failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

//PRINT LAST NAME

for (uint8_t i = 0; i < 16; i++) {

Serial.write(buffer2[i] );

}

//----------------------------------------

Serial.println(F("\n**End Reading**\n"));

delay(1000); //change value if you want to read cards faster

mfrc522.PICC_HaltA();

mfrc522.PCD_StopCrypto1();

}

--------------------------------------------------------------------------------------------

Write Prg :

#include <SPI.h>

#include <MFRC522.h>

#define RST_PIN         9           // Configurable, see typical pin layout above

#define SS_PIN          10          // Configurable, see typical pin layout above

MFRC522 mfrc522(SS_PIN, RST_PIN);   // Create MFRC522 instance

void setup() {

Serial.begin(9600);        // Initialize serial communications with the PC

SPI.begin();               // Init SPI bus

mfrc522.PCD_Init();        // Init MFRC522 card

Serial.println(F("Write personal data on a MIFARE PICC "));

}

void loop() {

// Prepare key - all keys are set to FFFFFFFFFFFFh at chip delivery from the factory.

MFRC522::MIFARE_Key key;

for (byte i = 0; i < 6; i++) key.keyByte[i] = 0xFF;

// Reset the loop if no new card present on the sensor/reader. This saves the entire process when idle.

if ( ! mfrc522.PICC_IsNewCardPresent()) {

return;

}

// Select one of the cards

if ( ! mfrc522.PICC_ReadCardSerial()) {

return;

}

Serial.print(F("Card UID:"));    //Dump UID

for (byte i = 0; i < mfrc522.uid.size; i++) {

Serial.print(mfrc522.uid.uidByte[i] < 0x10 ? " 0" : " ");

Serial.print(mfrc522.uid.uidByte[i], HEX);

}

Serial.print(F(" PICC type: "));   // Dump PICC type

MFRC522::PICC_Type piccType = mfrc522.PICC_GetType(mfrc522.uid.sak);

Serial.println(mfrc522.PICC_GetTypeName(piccType));

byte buffer[34];

byte block;

MFRC522::StatusCode status;

byte len;

Serial.setTimeout(20000L) ;     // wait until 20 seconds for input from serial

// Ask personal data: Family name

Serial.println(F("Type Family name, ending with #"));

len = Serial.readBytesUntil('#', (char *) buffer, 30) ; // read family name from serial

for (byte i = len; i < 30; i++) buffer[i] = ' ';     // pad with spaces

block = 1;

//Serial.println(F("Authenticating using key A..."));

status = mfrc522.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, &key, &(mfrc522.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("PCD_Authenticate() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

else Serial.println(F("PCD_Authenticate() success: "));

// Write block

status = mfrc522.MIFARE_Write(block, buffer, 16);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("MIFARE_Write() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

else Serial.println(F("MIFARE_Write() success: "));

block = 2;

//Serial.println(F("Authenticating using key A..."));

status = mfrc522.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, &key, &(mfrc522.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("PCD_Authenticate() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

// Write block

status = mfrc522.MIFARE_Write(block, &buffer[16], 16);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("MIFARE_Write() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

else Serial.println(F("MIFARE_Write() success: "));

// Ask personal data: First name

Serial.println(F("Type First name, ending with #"));

len = Serial.readBytesUntil('#', (char *) buffer, 20) ; // read first name from serial

for (byte i = len; i < 20; i++) buffer[i] = ' ';     // pad with spaces

block = 4;

//Serial.println(F("Authenticating using key A..."));

status = mfrc522.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, &key, &(mfrc522.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("PCD_Authenticate() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

// Write block

status = mfrc522.MIFARE_Write(block, buffer, 16);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("MIFARE_Write() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

else Serial.println(F("MIFARE_Write() success: "));

block = 5;

//Serial.println(F("Authenticating using key A..."));

status = mfrc522.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, &key, &(mfrc522.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("PCD_Authenticate() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

// Write block

status = mfrc522.MIFARE_Write(block, &buffer[16], 16);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("MIFARE_Write() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

else Serial.println(F("MIFARE_Write() success: "));

Serial.println(" ");

mfrc522.PICC_HaltA(); // Halt PICC

mfrc522.PCD_StopCrypto1();  // Stop encryption on PCD

}

Week-5

#include <DHT.h>

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

dht.begin();

}

void loop() {

float h = dht.readHumidity();

float t = dht.readTemperature();

float f = dht.readTemperature(true);

if (isnan(h) || isnan(t) || isnan(f)) {

Serial.println("Failed to read DHT");

return;

}

float hi = dht.computeHeatIndex(f, h);

float hiC = dht.computeHeatIndex(t, h, false);

Serial.print("Humidity: ");

Serial.print(h);

Serial.print(" %");

Serial.print(" Temperature: ");

Serial.print(t);

Serial.print(" °C ");

Serial.print(f);

Serial.print(" °F ");

Serial.print(" Heat index: ");

Serial.print(hiC);

Serial.print(" °C ");

Serial.print(hi);

Serial.println(" °F");

}

Week-6

#include <ESP8266WiFi.h>

String apiKey = "";

const char* ssid = "cmrith";

const char* pass = "1234";

const char* server = "api.thingspeak.com";

#define IRpin D4

WiFiClient client;

int value;

void setup()

{

Serial.begin(115200);

pinMode(IRpin, INPUT);

delay(1000);

Serial.begin("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, pass);

while (WiFi.status() != WL_CONNECTED)

{

delay(2000);

Serial.print(".");

}

Serial.println(" ");

Serial.println("WiFi Connected");

}

void loop()

{

value = digitalRead(IRpin);

Serial.println(value);

if (value == 0)

{

Serial.print("object detected");

}

else

{

Serial.print("no object detected");

}

if (client.connect(server, 80))

{

String postStr = apiKey;

postStr += "&field1=";

postStr += String(value);

postStr += "\r\n\r\n";

client.print("POST /update HTTP/1.1\n");

client.print("Host: api.thingspeak.com\n");

client.print("connection: close\n");

client.print("X-THINGSPEAKAPIKEY: " + apiKey + "\n");

client.print("Content-Type: application/x-www-form-urlencoded\n");

client.print("Content-Length: ");

client.print(postStr.length());

client.print("\n\n");

client.print(postStr);

client.stop();

}

Serial.println("waiting...");

delay(1000);

}

Week-7

#include <ESP8266WiFi.h>

#include <DHT.h>

const char* ssid = "your_wifi_name";

const char* password = "your_password";

String apiKey = "YOUR_API_KEY";

const char* server = "api.thingspeak.com";

#define DHTPIN D3

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

WiFiClient client;

void setup() {

Serial.begin(115200);

dht.begin();

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("WiFi connected");

}

void loop() {

float h = dht.readHumidity();

float t = dht.readTemperature();

if (isnan(h) || isnan(t)) {

Serial.println("Sensor Error");

return;

}

if (client.connect(server, 80)) {

String postStr = apiKey;

postStr += "&field1=" + String(t);

postStr += "&field2=" + String(h);

postStr += "\r\n\r\n";

client.print("POST /update HTTP/1.1\n");

client.print("Host: api.thingspeak.com\n");

client.print("Connection: close\n");

client.print("X-THINGSPEAKAPIKEY: " + apiKey + "\n");

client.print("Content-Type: application/x-www-form-urlencoded\n");

client.print("Content-Length: ");

client.print(postStr.length());

client.print("\n\n");

client.print(postStr);

Serial.print("Temp: ");

Serial.print(t);

Serial.print(" °C  Humidity: ");

Serial.println(h);

}

client.stop();

delay(15000);

}

Week-8

#include <ESP8266WiFi.h>

#include <DHT.h>

const char* ssid = "your_wifi_name";

const char* password = "your_password";

String apiKey = "YOUR_API_KEY";

const char* server = "api.thingspeak.com";

#define DHTPIN D3

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

WiFiClient client;

void setup() {

Serial.begin(115200);

dht.begin();

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("WiFi connected");

}

void loop() {

float h = dht.readHumidity();

float t = dht.readTemperature();

if (isnan(h) || isnan(t)) {

Serial.println("Sensor Error");

return;

}

if (client.connect(server, 80)) {

String postStr = apiKey;

postStr += "&field1=" + String(t);

postStr += "&field2=" + String(h);

postStr += "\r\n\r\n";

client.print("POST /update HTTP/1.1\n");

client.print("Host: api.thingspeak.com\n");

client.print("Connection: close\n");

client.print("X-THINGSPEAKAPIKEY: " + apiKey + "\n");

client.print("Content-Type: application/x-www-form-urlencoded\n");

client.print("Content-Length: ");

client.print(postStr.length());

client.print("\n\n");

client.print(postStr);

Serial.print("Temp: ");

Serial.print(t);

Serial.print(" °C  Humidity: ");

Serial.println(h);

}

client.stop();

delay(15000);

}

Week-9

#include <ESP8266WiFi.h>

#include <DHT.h>

// WiFi Credentials

const char* ssid = "your_wifi_name";

const char* password = "your_password";

// DHT Sensor Setup

#define DHTPIN D3

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

// Create TCP Server

WiFiServer server(80);

void setup() {

Serial.begin(115200);

delay(100);

// Start DHT Sensor

dht.begin();

// Connect to WiFi

Serial.println("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.print(".");

}

Serial.println("\nConnected!");

Serial.println(WiFi.localIP());

// Start Server

server.begin();

}

void loop() {

WiFiClient client = server.available();

if (!client) {

return;

}

Serial.println("Client connected");

// Read DHT Data

float humidity = dht.readHumidity();

float temperature = dht.readTemperature();

// Check if reading failed

if (isnan(humidity) || isnan(temperature)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

// Send Data to Client

client.print("Temperature: ");

client.print(temperature);

client.print(" °C\n");

client.print("Humidity: ");

client.print(humidity);

client.print(" %\n");

delay(2000);

client.stop();

Serial.println("Client disconnected");

}

Week-10

#include <ESP8266WiFi.h>

#include <WiFiUdp.h>

#include <DHT.h>

// WiFi Credentials

const char* ssid = "your_wifi_name";

const char* password = "your_password";

// UDP setup

WiFiUDP udp;

const char* udpAddress = "192.168.1.100";   // Receiver IP

const int udpPort = 4210;

// DHT Sensor

#define DHTPIN D3

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

Serial.println();

Serial.println("Connecting to WiFi");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(2000);

Serial.print(".");

}

Serial.println();

Serial.println("WiFi connected");

dht.begin();

}

void loop() {

delay(2000);

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

// Check sensor error

if (isnan(temperature) || isnan(humidity)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

Serial.print("Temperature: ");

Serial.println(temperature);

Serial.print("Humidity: ");

Serial.println(humidity);

Serial.println("Sending data over UDP");

udp.beginPacket(udpAddress, udpPort);

udp.print("Temperature: ");

udp.print(temperature);

udp.print(" C, ");

udp.print("Humidity: ");

udp.print(humidity);

udp.print(" %");

udp.endPacket();

Serial.println("Data sent");

}